TABLE OF CONTENTS

port-handler/port_handler

port-handler/port_handler                           port-handler/port_handler

NAME

port_handler -- standard serial/parallel/printer port handler.

SYNOPSIS

Open("PRT:[RAW/TRANSPARENT/UNIT=<Unit number>]",...);

Open("SER:[BAUD=<speed>/CONTROL=<settings>/UNIT=<Unit number>]",...);

Open("PAR:[UNIT=<Unit number>]",...);

FUNCTION

Port-handler allows AmigaDOS to interface to a standard parallel

device, a standard printer device, and a standard serial device.

Accessing "PAR:" connects AmigaDOS to the system's parallel port.

By default this will use parallel.device, unit 0. To use a

different unit number, such as 1, specify "PAR:UNIT=1" instead.

Accessing "PRT:" connects AmigaDOS to the system's printer. Data

written to PRT: is processed by the Amiga's printer device, handling

conversions of ANSI sequences, and sent to the printer. Accessing

"PRT:RAW" instead causes carriage return translation to be turned

off in the printer device while printing. To send data transparently

to the printer, without any translation or escape sequence substitution

whatsoever, use "PRT:TRANSPARENT". The TRANSPARENT and RAW switches

are mutually exclusive. By default "PRT:" will access printer.device,

unit 0. To use a different unit number, such as 1, specify "PRT:UNIT=1"

instead. The keywords can be combined, such as in "PRT:RAW/UNIT=1".

Accessing "SER:" connects AmigaDOS to the system's default serial port.

Following SER:, the baud rate and some control information can be

supplied. This is done in the form "SER:baud/control" where baud is

a number indicating the baud rate, and where control is a 3 letter

sequence denoting the number of read/write bits, the parity, and the

number of stop bits. For example, "SER:9600/8N1" connects to the

serial port, sets the baud rate to 9600, with 8 bit data, no parity,

and 1 stop bit. The possible control settings are:

1st character: 7 or 8

2nd character: N (No parity), O (Odd parity), E (Even parity)

M (Mark parity), S (Space parity)

3rd character: 1 or 2

Specifying no baud rate or control values when accessing SER: gives you

the values set in Serial preferences.

To override the default unit number, use the UNIT keyword, as with the

PRT: and PAR: devices described above. For example, to access unit 1

with 38400 bps you could use "SER:38400/UNIT=1".

PACKETS

ACTION_FINDINPUT

ACTION_FINDOUTPUT

ACTION_FINDUPDATE

ACTION_READ

ACTION_WRITE

ACTION_END

ACTION_IS_FILESYSTEM

MOUNTLIST ENTRIES

dos.library automatically mounts PAR:, PRT:, and SER: upon

system boot up. The mount entries used by dos.library are equivalent

to:

SER:    Handler   = L:Port-Handler

Priority  = 5

StackSize = 800

GlobVec   = 1

Startup   = 0

#

PAR:    Handler   = L:Port-Handler

Priority  = 5

StackSize = 800

GlobVec   = 1

Startup   = 1

#

PRT:    Handler   = L:Port-Handler

Priority  = 5

StackSize = 1000

GlobVec   = 1

Startup   = 2

#

Starting with V40, you can also provide disk-based mount entries

which let you use port-handler as a serial handler on various devices

and units. The form of these mount entries is:

SER0:   EHandler  = L:Port-Handler

Priority  = 5

StackSize = 800

GlobVec   = 1

Device    = serial.device

Unit      = 0

Flags     = 0

Control   = "8N1"

Baud      = 9600

#

The "Device", "Unit" and "Flags" keywords define the parameters

that the handler uses for OpenDevice(). "Baud" and "Control"

define the default values for baud rate and control information for

whenever that serial handler is accessed by name only, without

explicit baud rate and control information specified.

NOTE

Prior to V40, the baud rate and control information for SER: were

only recognized if an A2232 multi-serial card was installed in the

system. Starting with V40, these values are always recognized.

The TRANSPARENT, UNIT, BAUD and CONTROL keywords were introduced

with Port-Handler V44. For the different devices, the argument

templates are:

PRT:RAW/S,TRANSPARENT/S,UNIT/K/N

SER:BAUD/N,UNIT/K/N,CONTROL/F

PAR:UNIT/K/N

Instead of blank spaces, keywords must be separated by slashes ("/").