TABLE OF CONTENTS

parallel.device/CMD_CLEAR

parallel.device/CMD_FLUSH

parallel.device/CMD_READ

parallel.device/CMD_RESET

parallel.device/CMD_START

parallel.device/CMD_STOP

parallel.device/CMD_WRITE

parallel.device/OpenDevice

parallel.device/PDCMD_QUERY

parallel.device/PDCMD_SETPARAMS

parallel.device/CMD_CLEAR                           parallel.device/CMD_CLEAR

NAME

Clear -- clear the parallel port buffer

FUNCTION

This command just RTS's (no buffer to clear)

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_CLEAR (05)

parallel.device/CMD_FLUSH                           parallel.device/CMD_FLUSH

NAME

Flush -- clear all queued I/O requests for the parallel port

FUNCTION

This command purges the read and write request queues for the

parallel device. The currently active request is not purged.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_FLUSH (08)

parallel.device/CMD_READ                             parallel.device/CMD_READ

NAME

Read -- read input from parallel port

FUNCTION

This command causes a stream of characters to be read from the

parallel I/O register. The number of characters is specified in

io_Length. The EOF and EOL modes are supported, but be warned that

using these modes can result in a buffer overflow if the proper

EOL or EOF character is not received in time. These modes should

be used only when the sender and receiver have been designed to

cooperate. A safety guard can be implemented to EOF by setting

io_Length to a maximum allowed value. That cannot be done with EOL

since the EOL mode is identified by io_Length=-1.

The parallel.device has no internal buffer; if no read request has

been made, pending input (i.e. handshake request) is not

acknowledged.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_READ (02)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to receive.

io_Data         pointer where to put the data.

RESULTS

io_Error -- if the Read succeded, then io_Error will be null.

If the Read failed, then io_Error will contain an error code.

SEE ALSO

parallel.device/PDCMD_SETPARAMS

parallel.device/CMD_RESET                           parallel.device/CMD_RESET

NAME

Reset -- reinitializes the parallel device

FUNCTION

This command resets the parallel device to its freshly initialized

condition. It aborts all I/O requests both queued and current and

sets the devices's flags and parameters to their boot-up time

default values. At boot-up time the PTermArray is random, and it

will be so also here.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_RESET (01)

RESULTS

Error -- if the Reset succeded, then io_Error will be null.

if the Reset failed, then the io_Error will be non-zero.

parallel.device/CMD_START                           parallel.device/CMD_START

NAME

Start -- restart paused I/O over the parallel port

FUNCTION

This command restarts the current I/O activity on the parallel

port by reactivating the handshaking sequence.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_START (07)

SEE ALSO

parallel.device/CMD_STOP

parallel.device/CMD_STOP                             parallel.device/CMD_STOP

NAME

Stop -- pause current activity on the parallel device

FUNCTION

This command halts the current I/O activity on the parallel

device by discontinuing the handshaking sequence. The stop and

start commands may not be nested.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_STOP (06)

SEE ALSO

parallel.device/CMD_START

parallel.device/CMD_WRITE                           parallel.device/CMD_WRITE

NAME

Write -- send output to parallel port

FUNCTION

This command causes a stream of characters to be written to the

parallel output register. The number of characters is specified in

io_Length, unless -1 is used, in which case output is sent until

a zero byte occurs in the data. This is independent of, and may be

used simultaneously with setting the EOFMODE in io_ParFlags and using

the PTermArray to terminate the read or write.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_WRITE (03)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to transmit, or if set

to -1 send until zero byte encountered

io_Data         pointer to block of data to transmit

RESULTS

io_Error -- If the Write succeded, then io_Error will be null.

If the Write failed, then io_Error will contain an error code.

SEE ALSO

parallel.device/PDCMD_SETPARAMS

parallel.device/OpenDevice                         parallel.device/OpenDevice

NAME

Open -- a request to open the parallel port

SYNOPSIS

error = OpenDevice("parallel.device", unit, ioExtPar, flags)

D0                  A0                D0    A1        D1

FUNCTION

This function allows the requestor software access to the parallel

device.  Unless the shared-access bit (bit 5 of io_ParFlags) is

set, exclusive use is granted and no other access is allowed

until the owner closes the device.

A FAST_MODE, can be specified (bit 3 of io_Parflags) to speed up

transfers to high-speed printers. Rather than waiting for the printer

to acknowledge a character using the *ACK interrupt, this mode will

send out data as long as the BUSY signal is low. The printer must be

able to raise the BUSY signal within 3 micro-seconds on A2630s,

otherwise data will be lost. Should be used only in an exclusive-

access Open().

A SLOWMODE mode can be specified (bit 4 of io_ParFlags) when very

slow printers are used. If the printer acknowledges data at less

than 5000 bytes per second, then this mode will actually save CPU

time, although it consumes much more with high-speed printers.

The PTermArray of the ioExtPar is initialized only if the EOFMODE

bit (bit 1 of io_ParFlags) is set. The PTermArray can be further

modified using the PDCMD_SETPARAMS command.

INPUTS

"parallel.device" - a pointer to literal string "parallel.device"

unit - Must be zero for future compatibility

ioExtPar - pointer to an IO Request block of structure IOExtPar to

be initialized by the Open() function. The io_ParFlags

field must be set as desired.

flags - Must be zero for future compatibility

RESULTS

d0 -- same as io_Error

io_Error -- if the Open succeded, then io_Error will be null.

If the Open failed, then io_Error will be non-zero.

SEE ALSO

exec/CloseDevice()

parallel.device/PDCMD_QUERY                       parallel.device/PDCMD_QUERY

NAME

Query -- query parallel port/line status

FUNCTION

This command return the status of the parallel port lines and

registers.

IO REQUEST

io_Message      must have mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      PDCMD_QUERY (09)

RESULTS

io_Status        BIT  ACTIVE  FUNCTION

0     high   printer busy toggle (offline)

1     high   paper out

2     high   printer selected on the A1000

printer selected & serial "Ring

Indicator" on the A500/A2000

Use care when making cables.

3      -     read=0,write=1

4-7            reserved

BUGS

In a earlier version of this AutoDoc, BUSY and PSEL were reversed.

The function has always been correct.

parallel.device/PDCMD_SETPARAMS               parallel.device/PDCMD_SETPARAMS

NAME

SetParams -- change parameters for the parallel device

FUNCTION

This command allows the caller to change the EOFMODE parameter for

the parallel port device. It will disallow changes if any reads or

writes are active or queued.

The PARB_EOFMODE bit of io_ParFlags controlls whether the

io_PTermArray is to be used as an additional termination criteria

for reads and writes.  It may be set directly without a call to

SetParams, setting it here performs the additional service of

copying the PTermArray into the device default array which is used

as the initial array for subsequent device opens. The Shared bit

can be changed here, and overrides the current device access mode

set at OpenDevice time.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       preset by OpenDevice

io_Unit         preset by OpenDevice

io_Command      PDCMD_SETPARAMS (0A)

NOTE that the following fields of your IORequest

are filled by Open to reflect the parallel device's

current configuration.

io_PExtFlags    must be set to zero, unless used

io_ParFlags     see definition in parallel.i or parallel.h

NOTE that x00 yields exclusive access, PTermArray

inactive.

io_PTermArray   ASCII descending-ordered 8-byte array of

termination characters. If less than 8 chars

used, fill out array w/lowest valid value.

Terminators are used only if EOFMODE bit of

io_Parflags is set. (e.g. x512F040303030303 )

This field is filled on OpenDevice only if the

EOFMODE bit is set.

RESULTS

io_Error -- if the SetParams succeded, then io_Error will be null.

if the SetParams failed, then io_Error will be non-zero.