Simplification of expressions

involving exponentials and surds

Rainer Schoepf

Konrad-Zuse-Zentrum fuer Informationstechnik Berlin

Heilbronner Str. 10

W-1000 Berlin 31

Federal Republic of Germany

Email:  Schoepf@sc.ZIB-Berlin.de

This  short  note describes  a  new  REDUCE operator  that  tries  to

simplify expressions  that contain exponentials and  surds.  It  does

this by applying algebraic relations between these.

The operator is `EXPTSIMP'. Usage is

EXPTSIMP(EXP:exprn):exprn

The result is an equivalent expression.

Method.   The  basic idea  is to  replace every  surd or  exponential

by  a variable  and  add a  equations defining  the  variable.    For

the  resulting  polynomial  system is  the  Groebner  basis  is  then

calculated, and the surds and exponentials are substituted  back into

the result, leading  to a possibly simpler  expression.  Due to  this

method, no assumptions about a specific choice of branches  are made,

with the exception that  multiple occurences of the same surd  always

denote the same branch.

1