%N smagpplt

%D Plot,smooth and fit to gain and bandpass.

%P jhz

%: calibration

%B

SmaGpPlt is a MIRIAD task which plots and lists antenna gain and

bandbass. The plot for gains is against time. The

plot for bandpass is against the frequency. SmaGpPlt provides

options to moving smooth or orthogal polynomial fit to the

gain/bandpass solutions and to over write the old gain/bandpass

tables with the smoothed or polynomial fit results. In addition,

SmaGpPlt allows to compare bandpass solutions in two vis files.

SmaGpPlt can also merge two gain tables computed (using either

SmaMfCal or MfCal from two calibrators that were observed

interleavely.

%A vis

The name of the input data-set. This will normally be a visibility

data-set. No default. If two files are given, options must be

either 'bandpass' or 'gains,merge'.

%A device

The PGPLOT plotting device to use. The default is no plot.

%A log

The log to give a listing of the gains and polarization terms. The

default is no log.

%A xaxis

This specifies the variable of the x-axis in plot when options

is given as 'bandpass':

chan     Plot/list channels.

freq     Plot/list frequencies.

If nothing is given, 'freq' is assumed.

For options=gains, only time (UT) of x-axis is supported.

%A yaxis

This specifies what is to be plotted/listed. Several values can be

given, separated by commas. Minimum match is used:

amp      Plot/list amplitudes. This is the default if the gains

are being plotted/listed, and nothing else is requested.

phase    Plot/list phases.

real     Plot/list the real parts. This is the default if the

polarization terms are being plotted/listed, and nothing

else is requested.

imag     Plot/list the imaginary parts.

If nothing is given, 'amp' is assumed.

%A options

Some extra processing options can be given. Several values can be

given, separated by commas. Minimum match us used.

gains        Plot/list the gains vs time. This is the default if

nothing else is requested.

bandpass     Plot/list the bandpass shape vs frequency.

dots         Plot things as dots (rather than chunky circles).

dtime        Give time in days and fractions of a day. This is more

useful for listing files which are to be passed into

some other analysis or plotting tool.

wrap         Don't unwrap phase plots.

msmooth      Do moving average of the bandpass/gain curve using

the smooth parameters and replace old one.

weight       This gives weight in the orthogonal polynomial

least-square fit to the antenna gain

curves.

<1  => weight = 1, uniform weight.

1 or greater  => weight = the sqaure of amplitude

of 1/g, where g is a complex antenna gain

derived within a solutional interval.

The default is 1.

opolyfit     Do least-square fit to the bandpass/gain curves

with an orthogonal polynomial of degree n and replace

old one. Only one of the options is allowed in

the replacement of the gain/bandpass. Please choose

either msmooth or opolyfit.

ratio        do ratio (bp1/bp2) in comparison of two bandpasse

solutions if Keyword vis is given two input vis files.

The default is to compare the difference (bp2-bp1).

merge        merge two gain tables associated with the two

visibility files respectively. Two input vis

files must be given. The merged gain table

will be placed in the second input vis file

by overwritting on the old gain table.

xpass        transfers the bandpass solution from the image

side band (1st input file) to the target side

band (the 2nd input file) by reversing

the channel sequence of the solutions while

the frequency tables are kept unchanged.

vis must be given by two files containing

'bandpass' and 'freqs' produced from the

image bands (chunks) of the opposite

sidebands.

nofit        does not display polynomial fitting or

moving smooth curves on the gains/bandpass

solution plots.

%A filelabel

This gives an option to label the file name in the plot:

filelabel = -1, not to label the file name, the default;

filelabel =  0, label the file name to each of panel plots;

filelabel =  1, label the file name to the 1st panel in

the case of multiple panels.

%A smooth

This gives three parameters of moving smooth calculation of the

bandpass/gain curves

smooth(1) = K  parameter k giving the length 2k+1 of the averaging

interval; default is 3.

If smooth(1) =0, then polynomial fit will be

taken.

smooth(2) = L  order of the averaging polynomial l; default is 3.

smooth(3) = P  probability P for computing the confidence limits;

default is 0.9.

%A polyfit

polyfit(1) gives a degree of orthogonal polynomial in least-sqaure

fit to the bandpass/gain curves. Default is 0 (moving smooth).

If polyfit(1)=0, then the moving average is taken.

If polyfit(2)>0, then the Chi-square parameters as function

of a polynomial order  1 (linear) , 2 (parabolic),

3 (cubic), 4, 5 are reported for each antenna

and each spectral window. The minimum Chi-square corresponds to

the best fit in orthogonal polynomial regression.

%A nxy

Number of plots in the x and y directions. The default is 2,2.

%A select

A subset of the normal uv-selection parameters. They are not

entirely consisently used. Antenna and time selection is supported

for gains. Time selection is supported for delay and spectral

correction. Antenna and frequency selection is supported for

bandpasses. The default is to select everything.

%A yrange

The min and max range along the y axis of the plots. By default

gpplt autoscales. If the ``yrange'' parameter is given, then this

range is used on ALL plots. So it may not make much sense to

plot different sorts of quantities (e.g. amplitude and phases)

when explicitly giving the plot range.

%A dotsize

Allows users to choose a symbol (dot) size in a range between

1-201. The actual plotted dot size depends on the device

resolution. Other internal symbol selecting function

would be failed when this parameter is in use.

Default is to disable this function.