%N design

%D Interactive array design, uv coverage and beam

%P pjt

%: general

%B

DESIGN is an interactive program that allows you to add

pads and populate them with antennae to a site of your

selection.

For a selected design a synthesised beam is computed, and

the noise in the primary beam, exluding the central gaussian

part of the synthesised beam, is shown.

Commands in cursor mode are:

a   -- add an antenna (to this pad)

d   -- delete an antenna (from this pad)

p   -- add a new pad at this cursor position !!

(NOTE: Can't delete a pad position (*yet*)

c   -- check redundancy

(NOTE: not implemented yet)

x   -- execute a script to display uvcover and beam

UVTRACK, UVCOVER, INVERT, IMPLOT, DS etc.

r   -- redraw screen

l   -- list all antennae

?,h -- this help

q   -- quit

e   -- exit

%A pad

Name of the input file with pad locations.

The format of the text file is unformatted; the first two

columns are taken as the X and Y positions (a cartesion

system with origin at pad location 0N with units in Feet).

Default: $MIRCAT/hc.pad

%A ary

The name of a file containing the antenna positions for a

particular array.  This file can be used to initially select

a grouping of antennas.

Note:  If this file contains multiple configurations of a

group of antennas, that some of the cross-correlations present

in this simulation will not really be present in real data.

Default: no antennas initially selected.

%A antlist

List of antennae identifiers, numbered 1 through MAXPAD

to start the design with. Default: none supplied.

%A device

PGPLOT graphics devices (two can be given):

The first one is the one to edit the pads, it better be

an interactive device. The second device is optional, and

will be the same as the first one if not supplied; it it

used for the spawned script, and does not have to be

an interactive device.

Note: The first device needs to be interactive device.

%A minmax

Site Coverage, in feet, for displaying the data.

Four numbers need to be supplied: umin, umax, vmin, vmax.

Default: -520,520,-20,600 (useful for the basic T at

Hat Creek).

Eventually we'll allow arbitrary units etc.

%A vis

Output visibility dataset to be used. No default.

Maps and beams are stored as ``items'' within this dataset.

NOTE: If the dataset already existed, it will be deleted

without any user intervention using RMDATA.

%A imsize

The size of the output image in pixels, for INVERT. Only one

value can be given (i.e. a square image will be produced).

INVERT increases the sizes to the next power of two (if

they were not initially a power of 2). Default: 256.

%A cell

Image cell size, in arcsec, for INVERT. Only one value can be

given. Default: 1.0

%A box

Half the box size, in arcsec, that is used by HISTO to compute

statistics of the gaussian subtracted dirty beam. Must be less

than half the product of CELL and IMSIZE of course.

Default: 64.

%A server

This is used to specify the TV display device. It is giving in the

form:

type@server

or

type@device

The first form causes a connection to be made with a network

display server, whereas the second form uses local hardware

directly. For the network servers, the server process may have

to be started before invoking the Miriad task -- see below.

Currently supported types are:

xmtv          Miriad X server. `server' is the servers hostname.

(use "localhost" if you are not networked)

Normally the xmtv server will be the same host as

the X server. The user will normally invoke the xmtv

server before running any Miriad TV tasks, with

% xmtv

If the X server and xmtv server are on different

hosts, you may need to set the DISPLAY environment

variable on the xmtv server, and use xhost on the

X server.

ivas          IVAS (VMS only). `device' is the VMS device name.

You will require an IIS IVAS for this.

ivserver      IVAS server. `server' is the servers hostname.

Servers are currently installed on

castor.astro.uiuc.edu and bkyast.berkeley.edu.

msss          Miriad Sun Screen server. `server' is the serving

Sun, which can be any Sun with a console. The user

will normally invoke the server before running

any Miriad TV tasks, with

% msss

mxas          Miriad X server. `server' is the servers hostname.

This is an inferior server to xmtv.

If this keyword is present, the server will be used to display the

beam; the Maryland specific task 'mirsao' will be used otherwise.