%N velcolor

%D Make an RGB image cube to display velocity as color.

%P mchw

%: image analysis

%B

VELCOLOR makes a three-plane, red-green-blue, Miriad image cube

that can be used to generate a colour display of the third axis

of a data cube, typically velocity.  The RGB planes must be

superposed in a suitable viewer.

The algorithm weights the channels so that the apparent

intensity of the superposed RGB planes is independent of color.

For more details see Heiles & Jenkins (1976), A&A, 46, 33.

%A in

The input image.  No default.

%A region

The region of the input image to be used.  The 3rd axis region

determines the range from blue to red.  See documentation on

region for help of how to specify this.  Only the bounding box

is supported.

%A pivot

Center channel of input image for output green image.

Default is 0.4*trc+0.6*blc

%A out

The output RGB cube.  No default.

%A clip

Two values.  Exclude pixels with values in the range clip(1) to

clip(2).  If only one value is given, then exclude -abs(clip) to

+abs(clip).