%N invert

%D Transform multi-pointing visibility data into a map

%P rjs

%: map making

%B

INVERT is a MIRIAD task that forms images from visibilities.

INVERT can form continuum images or spectral line cubes.  It can

generate images/cubes for several polarisations, as well as

handling multi-frequency synthesis and mosaicing observations.

INVERT can also form complex-valued images from non-Hermitian

data (e.g. holography data).  Appropriate point-spread functions

can also be generated.

%A vis

Input visibility data files.  Several files can be given.

No default.

%A map

Output map (image) file name.  Each output file consists of a

single polarization/Stokes parameter.  If several different

pols/Stokes images are being made, then several file names

should be given.  No default.

%A beam

Output beam (point-spread function) file name.  The default is

not to make a beam.

%A imsize

The size of the output dataset.  The default is to image out to

primary beam half power points.  For options=mosaic, an image of

this size is made for each pointing before a linear mosaic

operation is performed.

%A cell

Image cell size, in arcsec.  If two values are given, they give

the RA and DEC cell sizes.  If only one value is given, the

cells are made square.  The default is about one third of the

resolution of the resultant images.

%A offset

When not mosaicing, this gives the sky position to shift to the

center of the output images.  The position is specified as an

offset (in arcsec) from the observing center.  The default is to

perform no shifting.

When mosaicing, this gives the sky coordinate (RA and DEC) of

the reference pixel in the imaging process.  The value can be

given in the form hh:mm:ss,dd:mm:ss, or as decimal hours and

degrees.  INVERT applies appropriate shifts to make this

location fall on a pixel.  The default is a central observing

center.

%A fwhm

This determines a gaussian taper to apply to the visibility

data.  It specifies the FWHM of an image-domain gaussian --

tapering the visibility data is equivalent to convolving with

this image-domain gaussian.

Either one or two values can be given, in arcsec, being the FWHM

in the RA and DEC directions.  If only one value is given, the

taper is assumed to be symmetric.  The default is no taper.

The signal-to-noise ratio will be optimised in the output image

if this parameter is set to the FWHM of typical image features

of interest.

If you are more accustomed to giving this parameter in the uv

plane (as AIPS requires), then:

fwhm(image plane) = 182 / fwhm(uv plane)

where the image plane fwhm is measured in arcseconds, and the uv

plane fwhm is measured in kilowavelengths.

%A sup

Sidelobe suppression area, given in arcseconds.  This parameter

gives the area around a source where INVERT attempts to suppress

sidelobes.  Two values (for the RA and DEC directions

respectively) can be given.  If only one value is given, the

suppression area is made square.  The default is to suppress

sidelobes in an area as large as the field being mapped.

Note that uniform weighting can produce images with spuriously

high noise, especially for mfs imaging. It is recommended

to use robust~0.5 if sup is non zero or unset.

The suppression area is essentially an alternate way of

specifying the weighting scheme being used.  Suppressing

sidelobes in the entire field corresponds to uniform weighting

(so the default corresponds to uniform weighting).  Natural

weighting gives the best signal to noise ratio, at the expense

of no sidelobe suppression.  Natural weighting corresponds to

SUP=0.  Values between these extremes give a tradeoff between

signal to noise and sidelobe suppression, and roughly correspond

to AIPS "super-uniform" weighting. [A better way to move between

these extremes is to leave sup unset and vary the robust

parameter from -2 to 2.]

%A robust

Brigg's visibility weighting robustness parameter.  This

parameter can be used to down-weight excessive weight being

given to visibilities in relatively sparsely filled regions of

the $u-v$ plane when using uniform weighting.  Most useful

settings are in the range [-2,2], with values less than -2

corresponding to very little down-weighting, and values greater

than +2 reducing the weighting to natural weighting.

Sidelobe levels and beam-shape degrade with increasing values of

robustness, but the theoretical noise level will also decrease.

The default is no down-weighting (robust=-infinity).

%A line

Standard "line" parameter, with the normal defaults.  In

particular, the default is to image all channels.  See the help

on "line" for more information.

The "line" parameter consists of a string followed by up to

four numbers, viz:

linetype,nchan,start,width,step

where 'linetype' is one of "channel", "wide", "velocity" or

"felocity".

%A ref

Line type of the reference channel, specified in a similar to

the "line" parameter.  Specifically, it is in the form:

linetype,start,width

Before mapping, the visibility data are divided by the reference

channel.  The default is no reference channel.

%A select

This allows a subset of the uv data to be used in the mapping

process.  See the Users Manual for information on how to specify

this parameter.  The default is to use all data.

%A stokes

Standard polarisation/Stokes parameter selection.  See the help

on "stokes" for more information.  Several polarisations can be

given.  The default is "ii" (i.e. Stokes-I, given the assumption

that the source is unpolarised).

%A options

This gives extra processing options.  Several options can be

given (abbreviated to uniqueness), and separated by commas:

nocal     Do not apply gains table calibration to the data.

nopol     Do not apply polarisation leakage corrections.

nopass    Do not apply bandpass table calibration to the data.

double    Normally INVERT makes the beam patterns the same

size as the output image.  This option causes the

beam patterns to be twice as large.

systemp   Weight each visibility in inverse proportion to the

noise variance.  Normally visibilities are weighted

in proportion to integration time.  Weighting based

on the noise variance optimises the signal-to-noise

ratio (provided the measures of the system

temperature are reliable!).

fsystemp  Like systemp, but use frequency dependent Tsys.

You need to run atrecal before invert to create the

systempf variable containing the Tsys spectrum.

Atrecal requires autocorrelations to be present.

This option only works in combination with the

mfs option.

mfs       Perform multi-frequency synthesis.  The causes all

the channel data to be used in forming a single map.

The frequency dependence of the uv coordinate is

thus used to give better uv coverage and/or avoid

frequency smearing.  For this option to produce

useful maps, the intensity change over the frequency

band must be small.  Set the 'line' parameter to

select the channels that you wish to grid.

sdb       Generate the spectral dirty beam as well as the

normal beam, when MFS processing.  The default is

only to create the normal beam.  If the spectral

dirty beam is created, this is saved as an extra

plane in the beam dataset.

mosaic    Process multiple pointings, and generate a linear

mosaic of these pointings. For single pointings

to be combined with linmos you can use this to

specify a common reference position with the

offset parameter. Observations using OTF mosaicing

always need to specify this to ensure the moving

beam is handled properly.

imaginary Make imaginary image for non-Hermitian data

(holography).

amplitude Produce a image using the data amplitudes only.  The

phases of the data are set to zero.

phase     Produce an image using the data phase only.  The

amplitudes of the data are set to 1.

sin       Label the output map and beam as a SIN projection.

Default is NCP unless non-east-west baselines are

present or the field centre is within 3 deg of the

celestial equator (because NCP blows up near the

equator).  Note that this option simply changes

ctype1 and ctype2 in the header, the translation

only being correct to first order about the field

centre.  A similar result could be obtained by

running 'puthd' on the output map, e.g.

puthd in=<map>/ctype1 value=RA---SIN

puthd in=<map>/ctype2 value=DEC--SIN

and likewise for the beam.

%A mode

This determines the algorithm to be used in imaging.

Possible values are:

fft    The conventional grid-and-FFT approach.  This is the

default and by far the fastest.

dft    Use a discrete Fourier transform.  This avoids aliasing

but at a hugh time penalty.

median This uses a median approach.  This is generally robust

to bad data and sidelobes, has a even larger time

penalty and produces images that cannot be deconvolved.

NOTE: Dft and median modes are not supported with

options=mosaic.

%A slop

NOTE: This parameter should be used with caution!  See the Users

Guide for more information on its applicability.

When forming spectral cubes, INVERT normally insists that all

channels in a given visibility spectrum must be good before

accepting the spectrum for imaging.  This keyword allows this

rule to be relaxed.  It consists of two parts: a tolerance and

a method for replacing the bad channels.

The tolerance is a value between 0 and 1, giving the fraction of

channels that INVERT will tolerate as being bad before the

spectrum is totally discarded.  The default is 0, indicating

that INVERT will not tolerate any bad channels.  A value of 1

indicates that INVERT will accept a spectrum as long as there is

at least one good channel.

The replacement method is either the value `zero' or

'interpolate', indicating that the bad channels are either to be

replaced with 0, or to be estimated by linear interpolation of

two adjacent good channels.  See the Users Guide for the merits

and evils of the two approaches.  The default is 'zero'.