%N rmsfit

%D Non-linear Least Squares Fitting

%P mchw

%: misc, utility

%B

RMSFIT is a task that fits one of the following functions

with up to 6 parameters (a,b,c,..) to a table of values.

The first three functions use fixed column numbers:

1. Phase Structure Function.

z = a * x**b * sin(y)**c + d

For the task ATMOS. x, y and z are in column 5, 6 and 8 resp.

2.      z = a * x**b + c * y

x, y, and z in column 5, 4, and 8

3.      z = a * x**2 + b * x + c

x, z in column 1 and 2 respectively.

The column numbers must be specified for following functions:

Lines that begin with '#' are ignored. Giberish can appear in

unused columns. Columns are separated by blanks.

The maximum line length is 320 characters.

Column numbers for x, y, and z specified below.

4.      z = a * x**b + c

5.      z = a + b * x + c * x**2 + d * x**3

6. Comet model radial profile of surface density (brightness)

z = a/x * integral{exp[-x/b sec(theta)]} theta= -pi/2 to pi/2

7. Atmospheric opacity:

z = a + b*(1-exp(-c*x))  ! Tb = Trx + Tsky(1-exp(-airmass*tau0))

8. Drift scan analysis

z = a * (exp(-b*(x-c)**2) - exp(-b*(x-d)**2))

9. Primary beam pattern polynomial fit.

z = 1 + a1*x**2 + a2*x**4 + ...         "x" is in units of arcmin*GHz.

10. Primary beam pattern inverse polynomial fit.

z = 1/(1 + a1*x**2 + a2*x**4 + ... )     "x" is in units of arcmin*GHz.

11. Primary beam pattern cos**6 fit.

z = a1*cos(x)**6  "x" is in units of arcmin*GHz.

12. line length fit:

z = a1*cos(2pi*a2*x + a3)  "x" is in MHz, a1 is amplitude,

a2 is the line length in microsec, and a3 is the phase offset.

%A opt

specifies which function is to be fitted. Default opt=1.

%A col

column numbers for x, y, z  variables. Three values. Default=0,0,0

Setting a column number = 0, sets the corresponding variable=1,

and modifies the equation accordingly.

%A in

Input ascii file with columns of data. No default.

%A npts

Select the first ``npts'' from the input file.

Default reads the whole file or 2000 which is the maximum allowed.

%A par

Initial estimates for parameters (a,b,c,..).

An initial guess must be specified for each parameter used.

Default=0,0,0,0,0,0 may not be a good starting guess.

%A nfit

Number of paramters to fit. Counts from a,b,c,d onwards,

e.g. nfit=3 would only fit a,b,c and leave d fixed.

Default: 2

%A log

Output logfile. It will contain the results, a histogram

with residuals and all selected measurements, the fit, and residuals.

Default: print results and histogram only to terminal.