%N uvsector

%D Flag a sector of UV data.

%P rjs

%: calibration

%B

UVSECTOR flags visibility data with a specific sector.

The sector can be specified either by position angle, or as

an hour angle (East-West array assumed) or by way of a stripe

direction in an image. The latter form

will be useful when a stripe is seen in an image (implying

bad visibility data), and you wish to flag out the visibility

data that could have produced this stripe.

%A vis

The visibility file to flag. Several files can be given.

Wildcard expansion is supported. The default is no file.

%A select

Extra selection criteria. In addition to being in the appropriate

sector, the data must satisfy the selection criteria before

it is flagged.

%A angle

This defines the angle of the axis of the sector, if the sector is

given by a position angle or hour angle (for an EW array). It

consists of two values, the first being either the string "uvangle"

or "hangle" (for position angle and hour angle, respectively). The

second gives the angle in degrees or hours (for position angle

and hour angle respectively). Position angle is defined as degrees

clockwise from the v axis (which is consistent with the uvplt

definition).

%A in

If the sectors position angle is given by a stripe direction,

this gives the name of the image to associate with the stripe.

%A region

A long, thin, region of the image given above, running in the

direction of a stripe. This parameter must be given if you

are defining an hour angle by a stripe direction. Normally

you will generate this region using CGCURS's region option,

and generate a region along the ridge of a stripe.

%A width

The full width of the sector, in degrees. This must be in the

range (0,180). The default is 5 degrees.

%A options

Extra processing options. Several can be given, separated by commas.

Minimum match is used. There is currently only one option.

noapply    Report on what would be done, but do not apply the

flags.