%N at_mfcal

%D Multifrequency antenna and passband calibration.

%P rjs

%: calibration

%B

MfCal is a Miriad task which determines calibration corrections

(antenna gains, delay terms and passband responses) from a

multi-frequency observation.  The delays and passband are

determined from an average of all the selected data.  The gains

are worked out periodically depending upon the user set interval.

%A vis

Input visibility data file. No default. This can (indeed should)

contain multiple channels and spectral windows. The frequency

set-up can vary with time.

%A line

Standard line parameter, with standard defaults.

%A stokes

Stokes parameters to process. The default is to process the parallel

hand polarisations.

%A edge

The number of channels, at the edges of each spectral window, that

are to be dropped. Either one or two numbers can be given, being the

number of channels at the start and end of each spectral window to be

dropped. If only one number is given, then this number of channels

is dropped from both the start and end of each window. The default

value is 0.

%A select

Standard uv selection. The default is to select all data.

%A flux

Three numbers, giving the source flux density, a reference frequency

(in GHz) and the source spectral index. The flux and spectral index

are at the reference frequency. If no values are given, then MFCAL

checks whether the source is one of a set of known sources, and uses the

appropriate flux variation with frequency. Otherwise the default flux

is determined so that the rms gain amplitude is 1, and the default

spectral index is 0. The default reference frequency is the mean of

the frequencies in the input data. Also see the `oldflux' option.

%A refant

The reference antenna. Default is 3. The reference antenna needs

to be present throughout the observation. Any solution intervals

where the reference antenna is missing are discarded.

%A minants

The minimum number of antennae that must be present before a

solution is attempted. Default is 2.

%A interval

This gives one, two or three numbers, all given in minutes, being

used to determine the extents of the antenna gain and passband

calibration solution interval.

The first gives the maximum length of a solution interval. The

second gives the maximum gap size in a solution interval. A new solution

interval is started when either the max length is exceeded, or a

gap larger than the max gap size is encountered. The default max length is

5 minutes, and the default max gap size is the same as the max length.

The third number specifies the passband solution interval. The default is

to use a single passband solution for the entire observation. Each passband

solution interval will consist of one or more full gain solution intervals.

%A options

Extra processing options. Several values can be given, separated by

commas. Minimum match is used. Possible values are:

delay     Attempt to solve for the delay parameters. This can

be a large sink of CPU time. This option rarely works and

should be used with caution.

nopassol  Do not solve for bandpass shape. In this case if a bandpass

table is present in the visibility data-set, then it will

be applied to the data.

interpolate Interpolate (and extrapolate) via a spline fit (to

the real and imaginary parts) bandpass values for

channels with no solution. This is commonly used because

a set of channels are flagged, possibly because of RFI or

spectral features in the bandpass calibrator.  If

less than 50% of the channels are good, the

interpolation (extrapolation) is not done and those

channels will not have a bandpass solution

oldflux   This causes MFCAL to use a pre-August 1994 ATCA flux

density scale. See the help on "oldflux" for more

information.

%A tol

Solution convergence tolerance. Default is 0.001.