%N uvclip

%D Determine data to flag from channel-0 uv dataset

%P njt

%: uv analysis

%B

UVCLIP flags UV channel 0 data set by statistical clipping

%A vis

The input UV dataset name. No default.

%A select

This selects the data to be processed, using the standard uvselect

format. Default is all data.

%A options

noamp .... no visbility amplitude clipping

notrip ... no triple visibility phase closure clipping

doall .... include all data (flagged and unflagged)

noapply .. do not apply flagging

%A interval

Data clipping interval in minutes. The maximum interval between

data points for continuous clipping.  Must be greater than the

integration time.  The default is infinite.

%A clip

Clipping level.  Number of rms beyond which data is flagged.

Default is 3.

%A log

The list output file name. The default is the terminal.